Parada A Module 1 — The Basics

Module objective: Learn to recognize mathematical relationships, interpret graphs, convert units, and solve proportionality questions quickly and accurately. These skills recur throughout ultrasound physics, including Doppler, attenuation, pulse timing, transducers, and image optimization.

The topic sequence follows the foundational material associated with Edelman's chapter The Basics.

Part A — Concept review

1. Direct and inverse relationships

The SPI frequently tests whether you understand how one quantity changes when another quantity changes.

Direct relationship

Both quantities change in the same direction when other relevant variables remain constant.

Direct proportionality

Doubling x doubles y when y = kx.

Inverse relationship

One quantity increases while the other decreases, assuming their product remains constant.

Inverse proportionality

Critical distinction: An inverse relationship does not mean that one quantity is negative. It means that the quantities move in opposite directions under the stated conditions.

Examples you will use later:

| Relationship | Type | Condition |
| --- | --- | --- |
| Frequency and period | Inverse | (T=1/f |
| Frequency and wavelength | Inverse | Propagation speed is constant |
| PRF and PRP | Inverse | PRP=1/PRF |
| Frequency and propagation speed | Not generally proportional | Speed depends primarily on the medium |
| Amplitude and intensity | Square relationship | Same relevant wave and medium; intensity proportional to amplitude squared |

The last example is particularly important. If amplitude doubles, intensity increases fourfold, not twofold.

2. The metric system and common ultrasound units

| Quantity | Common unit | Equivalent |
| --- | --- | --- |
| Length | Meter (m) | 1 m = 100 cm |
| Length | Centimeter (cm) | 1 cm = 10 mm |
| Length | Millimeter (mm) | 1 mm = 1,000 µm |
| Time | Second (s) | 1 s = 1,000 ms |
| Time | Millisecond (ms) | 1 ms = 1,000 µs |
| Frequency | Hertz (Hz) | 1 kHz = 1,000 Hz |
| Frequency | Megahertz (MHz) | 1 MHz = 1,000,000 Hz |
| Speed | m/s | 1,540 m/s = 1.54 mm/µs |

This conversion is valuable because ultrasound calculations often combine distances in millimeters with travel times in microseconds.

3. Powers of ten and scientific notation

Exam trap: A correct formula with an incorrect power of ten can produce an answer that is off by a factor of 1,000 or 1,000,000.

4. Reading graphs

When a graph appears in a physics question, identify:

- What the horizontal axis represents.

- What the vertical axis represents.

- Whether the relationship is direct, inverse, constant, or more complex.

- Whether the question asks for a value, a trend, or a comparison.

A straight line through the origin indicates direct proportionality when the axes are linear. A downward-curving graph may indicate inverse proportionality. A horizontal line indicates that the vertical quantity remains constant as the horizontal quantity changes.

Do not confuse a steep slope with a large value: slope measures the rate of change, not the absolute value of the quantity.

Part B — Exam-level multiple-choice questions

Instructions: Answer all 15 questions before reviewing the explanations. Some questions are straightforward, while others are deliberately designed to expose common reasoning errors. Select the single best answer.

Practice set

0 / 15 answered

1. A variable is directly proportional to another variable. If the second variable increases by a factor of 4, what happens to the first?

A. It decreases to one-fourth.

B. It increases by a factor of 2.

C. It increases by a factor of 4.

D. It increases by a factor of 16.

Not answered — Answer: C

For direct proportionality, y = kx. Multiplying x by 4 multiplies y by 4. A is the inverse relationship; B confuses proportionality with square-root scaling; D confuses it with a square relationship.

Exam trap: Confusing direct proportion with inverse or square relationships.

2. The period of a wave decreases from 0.4 µs to 0.2 µs. What happens to frequency?

A. It decreases by half.

B. It doubles.

C. It remains unchanged.

D. It quadruples.

Not answered — Answer: B

Frequency and period are reciprocals: f = 1/T. Halving the period doubles frequency.

Exam trap: Remember that period is time per cycle, while frequency is cycles per second.

3. Which is the correct conversion of 7.5 MHz to hertz?

A. 7,500 Hz

B. 75,000 Hz

C. 750,000 Hz

D. 7,500,000 Hz

Not answered — Answer: D

1 MHz = 10⁶ Hz. Therefore 7.5 MHz = 7.5 × 10⁶ Hz = 7,500,000 Hz.

Exam trap: Confusing the prefixes kilo (10³) and mega (10⁶).

4. A distance is recorded as 0.035 m. What is this distance in millimeters?

A. 0.35 mm

B. 3.5 mm

C. 35 mm

D. 350 mm

Not answered — Answer: C

One meter contains 1,000 millimeters. 0.035 × 1,000 = 35 mm.

Exam trap: When converting meters to millimeters, move three decimal places to the right.

5. Which statement best describes the graph y = 5x?

A. y is inversely proportional to x.

B. y is directly proportional to x.

C. y is constant regardless of x.

D. y is proportional to the square of x.

Not answered — Answer: B

The equation has the form y = kx, where k = 5. The constant multiplier does not change the direct proportionality.

Exam trap: The coefficient changes the scale, not the type of relationship.

6. Which equation represents an inverse relationship between x and y?

A. y = 3x

B. y = x + 3

C. y = 3/x

D. y = x²

Not answered — Answer: C

An inverse relationship has the variable in the denominator when the product of the variables is constant: y = k/x.

Exam trap: A decreasing graph is not automatically an inverse-proportion graph; inspect the equation.

7. A quantity follows y = kx². If x doubles, what happens to y?

A. It doubles.

B. It triples.

C. It increases fourfold.

D. It decreases by half.

Not answered — Answer: C

Substitute 2x: y becomes k(2x)² = 4kx². The quantity increases fourfold.

Exam trap: The factor applied to the variable must also be squared.

8. Which value is equivalent to 250 µs?

A. 0.00025 s

B. 0.0025 s

C. 0.025 s

D. 0.25 s

Not answered — Answer: A

250 µs = 250 × 10⁻⁶ s = 0.00025 s.

Exam trap: Convert microseconds to seconds using 10⁻⁶, not 10⁻³.

9. A graph is a straight line that passes through the origin. What can you conclude if both axes are linear?

A. The variables must be inversely proportional.

B. The variables have a direct proportional relationship.

C. The variables must be unrelated.

D. The slope must equal 1.

Not answered — Answer: B

A straight line through the origin represents y = kx, which is direct proportionality. The slope may be any constant, not just 1.

Exam trap: A straight line does not have to be a 45-degree line or have slope 1.

10. Which statement about a horizontal line on a graph is correct?

A. The horizontal-axis variable is constant.

B. The vertical-axis variable is constant.

C. Both variables are constant.

D. The variables are inversely proportional.

Not answered — Answer: B

A horizontal line has the same vertical-axis value at every horizontal-axis position.

Exam trap: Read the axes rather than assuming which variable remains unchanged.

11. A student claims that because y = 2x + 5 is a straight-line equation, y is directly proportional to x. What is the best response?

A. Correct, because every straight line represents direct proportionality.

B. Incorrect, because direct proportionality requires the line to pass through the origin.

C. Correct, because the slope is positive.

D. Incorrect, because y decreases as x increases.

Not answered — Answer: B

Direct proportionality has the form y = kx and passes through the origin. The additional constant 5 creates a nonzero intercept.

Exam trap: A linear relationship is not always a direct proportion.

12. Which quantity is equivalent to 1.54 mm/µs when expressed in m/s?

A. 1.54 m/s

B. 154 m/s

C. 1,540 m/s

D. 154,000 m/s

Not answered — Answer: C

1.54 mm = 0.00154 m and 1 µs = 0.000001 s. Dividing gives 1,540 m/s.

Exam trap: Converting the numerator and denominator separately prevents errors involving metric prefixes.

13. A value changes from 20 to 50. What is the multiplicative factor of change?

A. 2.0

B. 2.5

C. 3.0

D. 30

Not answered — Answer: B

Divide the final value by the initial value: 50/20 = 2.5. The final value is 2.5 times the original.

Exam trap: The factor is not the absolute difference (30) or the percentage increase (150%).

14. A variable is inversely proportional to x. If x increases by 50%, what happens to the variable?

A. It decreases by exactly 50%.

B. It increases by 50%.

C. It becomes two-thirds of its original value.

D. It becomes 1.5 times its original value.

Not answered — Answer: C

A 50% increase makes x equal to 1.5 times its original value. For an inverse relationship, the other variable becomes 1/1.5 = 2/3 of its original value, a decrease of about 33.3%.

Exam trap: Inverse changes are not generally equal percentage changes in opposite directions.

15. An exam question states that intensity is proportional to amplitude squared. If amplitude decreases to one-half of its original value, what happens to intensity?

A. It decreases to one-half.

B. It decreases to one-quarter.

C. It doubles.

D. It decreases to one-eighth.

Not answered — Answer: B

Intensity ∝ amplitude². Halving amplitude gives (1/2)² = 1/4 of the original intensity.

Exam trap: Apply the exponent to the entire scaling factor, not just to the original value.

Reveal answer key and explanations

Your practice result

0 / 15

This is a learning checkpoint, not an official SPI passing-score conversion. Review all incorrect answers and any questions you guessed on.

Retry all questions

Part C — The deeper reasoning behind the answers

The most valuable lesson from this module is that the size of a change matters just as much as its direction.

Compare these three relationships:

| Relationship | If \(x\) doubles | If \(x\) increases by 50% |
| --- | --- | --- |
| \(y\propto x\) | \(y\) doubles | \(y\) increases by 50% |
| \(y\propto 1/x\) | \(y\) halves | \(y\) becomes \(2/3\) of its original value |
| \(y\propto x^2\) | \(y\) becomes 4 times larger | \(y\) becomes 2.25 times larger |

This distinction becomes essential in later modules. For example, ultrasound intensity is proportional to amplitude squared, while frequency and period are inversely related.

A reliable four-step exam method

1

Identify the equation

Determine which formula or proportional relationship connects the quantities.

2

Identify what is held constant

Ask whether the medium, propagation speed, or another relevant variable is unchanged.

3

Predict before reading the options

Decide whether the result should increase, decrease, remain constant, or change by a particular factor.

4

Check units and distractors

Eliminate answers with the wrong direction, incorrect scaling factor, or incompatible units.

Part D — Cross-topic connections

These foundational skills will reappear in later modules.

Connection 1: The Basics → Describing Sound

Foundational relationship

\[ f=\frac{1}{T} \]

If frequency doubles, period halves. This relationship is used when comparing transducers and interpreting wave parameters.

Connection 2: The Basics → Describing Pulsed Waves

Reciprocal relationship

\[ PRF=\frac{1}{PRP} \]

If PRF doubles, PRP halves. This will become important when studying depth, frame rate, and Doppler aliasing.

Connection 3: The Basics → Intensities

Square relationship

\[ I\propto A^2 \]

Doubling amplitude produces four times the intensity, assuming the relevant conditions remain constant. This is a common source of incorrect answers.

Connection 4: The Basics → Doppler

Multiple-variable equation

Doppler shift depends on transmitted frequency, blood velocity, propagation speed, and the cosine of the Doppler angle. Students must distinguish direct relationships from variables that have different effects.

Part E — Module 1 mastery checklist

Readiness check

0 of 8

I can distinguish direct proportionality from inverse proportionality.

I can identify a square relationship and apply its exponent correctly.

I can convert between meters, millimeters, micrometers, seconds, milliseconds, and microseconds.

I can convert MHz and kHz to Hz using powers of ten.

I can interpret a graph's axes, slope, intercept, and general trend.

I can distinguish a straight-line relationship from direct proportionality.

I can solve a proportionality question before looking at the answer choices.

I can explain why frequency-period and PRF-PRP are reciprocal relationships.

Keep practicing

Review any unchecked skill, especially if you missed a related question in the practice set.

Module 1 complete